Weather Station & Thermostat

Project Documentation

- Project Overview

This project implements a dual-function embedded system using an Arduino:

- A weather station that measures and displays ambient temperature

- A thermostat that compares ambient temperature with a user-defined reference and signals heating or cooling modes

The system is designed to be efficient, using low-level register manipulation instead of standard Arduino libraries.

- Features

- Temperature measurement with 0.1°C precision

- Display output on a 4-digit 7-segment display

- Dual operating modes:
• Weather Station Mode - displays ambient temperature
• Thermostat Mode - compares ambient and set temperatures

- Visual feedback using LEDs:
• Green LED - Heating mode
• Blue LED - Cooling mode

- Adjustable reference temperature using a potentiometer

3. Hardware Components

- NTC Thermistor

- Hall Effect Sensor (partially implemented)

- 2 LEDs

- 4-digit 7-segment display

- 2 Potentiometers

- Push Button

- Fan

4. System Functionality

4.1 Temperature Measurement
The temperature is measured using an NTC thermistor in a voltage divider. The analog voltage is converted into temperature using linear interpolation.

4.2 Display Control
The display is multiplexed using Timer1 interrupts, switching digits every millisecond.

4.3 Thermostat Logic

- If set temperature > ambient - Heating (green LED)

- If set temperature < ambient - Cooling (blue LED)

5. Software Implementation

- Written in Arduino C

- Uses direct register manipulation

- Timer interrupts for display

- Custom interpolation function

6. Limitations & Improvements

- Fan RPM monitoring not fully implemented

- Possible improvements:
• RPM control
• Better calibration
• LCD/OLED display

7. Conclusion
This project demonstrates an efficient embedded system combining sensing, display, and control logic.